# Report 2 - TinyML Wake Word Detection on XIAO ESP32S3 Sense

Student: Nisipeanu Ionut

# Compliance Note

The current deployed prototype uses two output classes: `wake_word` and `not_wake`. The Report 2 brief asks for classification with at least three classes. Therefore, the current working prototype satisfies the XIAO ESP32S3 Sense deployment requirement, real-time inference requirement, quantized TinyML deployment requirement, and live sensor requirement, but it is not strictly compliant with the three-class rule until the model is extended to a third output class such as `background_noise` or `unknown_speech`.

The shortest compliant extension is to retrain and deploy a three-class model with:

`wake_word`: the phrase "hey computer"

`unknown_speech`: spoken words that are not the wake phrase

`background_noise`: silence or room noise

# 1. Problem Definition and Dataset

The goal of this project is to detect a local wake phrase on a microcontroller. The physical device is a Seeed Studio XIAO ESP32S3 Sense, using its built-in PDM microphone as the live sensor. When the phrase "hey computer" is detected, the firmware sends an HTTP ping to a Python server running on the PC. The detection is done locally on the microcontroller using a quantized TensorFlow Lite Micro model.

The current deployed model is a binary classifier:

| Class | Meaning |
| --- | --- |
| wake_word | The user says "hey computer" |
| not_wake | Other speech, silence, or non-wake audio |

The dataset was collected directly from the XIAO microphone using a data-collection mode in the firmware. Each captured clip is uploaded over Wi-Fi to the PC server and stored as a WAV file. The old webcam microphone recordings are ignored by the training script because the acoustic characteristics were too different from the XIAO microphone.

Current XIAO-only dataset used by the latest training run:

| Source folder | Used as class | Files used |
| --- | --- | --- |
| dataset/raw/wake_word | wake_word | 18 |
| dataset/raw/unknown_speech | not_wake | 20 |

Audio settings:

Sample rate: 16000 Hz

Channels: mono

Bit depth: 16-bit PCM

Inference window length: 2.0 seconds

Feature count: 64 energy-envelope features per window

Camera subfolders are ignored during training

Insert setup photo here before submission: XIAO ESP32S3 Sense connected by USB-C, Arduino IDE serial monitor, and PC server terminal.

# 2. Model Architecture

The final deployed model uses compact audio energy features instead of raw audio samples. Raw 2-second audio windows contain 32000 samples, which was too high-dimensional for the small dataset and caused unstable or flat predictions. The model now converts each 2-second window into 64 energy features by splitting the audio into short frames, removing DC offset, and computing the mean absolute amplitude per frame.

Model architecture:

| Layer | Output shape | Parameters |
| --- | --- | --- |
| Input energy features | 64 | 0 |
| Dense, ReLU | 16 | 1040 |
| Dense, ReLU | 8 | 136 |
| Dense, Softmax | 2 | 18 |
| Total | - | 1194 |

Training configuration:

Framework: TensorFlow / Keras

Loss: sparse categorical cross-entropy

Optimizer: Adam

Learning rate: 0.01

Early stopping on validation accuracy

Data augmentation: random gain, small temporal shift, and low-amplitude white noise

Train/test split is done by source file to avoid leakage between windows from the same recording

# 3. Training Results

The latest training run used 32 training examples and 8 test examples after window selection, augmentation, and splitting. Training stopped after 13 epochs using early stopping.

Training output summary:

| Metric | Value |
| --- | --- |
| Test accuracy | 0.750 |
| Test loss | 0.645 |
| Train examples | 32 |
| Test examples | 8 |
| Epochs run | 13 |
| Keras model size | 39755 bytes |
| INT8 TFLite model size | 3536 bytes |

The earlier raw-audio model often produced nearly equal probabilities, for example `wake_word=0.500 not_wake=0.500`. This was fixed by switching to energy-envelope features and matching the same feature extraction on both the PC and the microcontroller.

# 4. Quantization

The trained Keras model was converted to a fully integer TensorFlow Lite model for microcontroller deployment.

Quantization details:

Converter: TensorFlow Lite Converter

Optimization: `tf.lite.Optimize.DEFAULT`

Representative dataset: selected training feature windows

Input type: int8

Output type: int8

Model file: `training/output/wake_word_model_int8.tflite`

Arduino header: `esp32_tinyml_wake_word/model_data.h`

Quantization is important because it reduces the model size and allows TensorFlow Lite Micro to run inference on the XIAO ESP32S3 Sense with limited memory.

# 5. TinyML Deployment

The firmware is implemented in Arduino C++ in `esp32_tinyml_wake_word/esp32_tinyml_wake_word.ino`. The generated model is compiled into the firmware through `model_data.h`.

Deployment pipeline:

Collect labeled audio clips from the XIAO microphone.

Save clips on the PC server under `dataset/raw`.

Train the local Keras model with `training/train_local_wake_word.py`.

Convert the model to int8 TensorFlow Lite.

Generate `model_data.h`.

Flash the Arduino sketch to the XIAO ESP32S3 Sense.

Run live inference from the PDM microphone.

Send an HTTP ping to the PC server when the wake score passes the threshold.

Firmware settings used during the successful live test:

| Setting | Value |
| --- | --- |
| Wake threshold | 0.55 |
| Listen phase | 3.0 seconds |
| Inference window | 2.0 seconds |
| Windows per listen phase | 2 |
| Cooldown after prediction | 5.0 seconds |
| Internal debounce | 3.0 seconds |
| PDM clock pin | 42 |
| PDM data pin | 41 |

The firmware evaluates rolling 2-second windows during each 3-second listen phase. This avoids missing the wake phrase if the user speaks immediately after the "Speak now" message.

Example successful serial/server output:

Best window: 1/2 Predictions: wake_word=0.711 not_wake=0.289
Wake word detected with confidence 0.711
Server status: 200
{"status":"ok","event":"wake_word","count":5,"score":"0.711","timestamp":"2026-05-21T00:47:02"}
Ping sent

# 6. Live Inference Results

The assignment requires at least 20 physical-device test predictions. Only the first three rows below are real results currently available from the serial monitor output. Rows marked TODO must be filled from additional live XIAO tests before final submission.

| Test | True label | Predicted label | Confidence | Result |
| --- | --- | --- | --- | --- |
| 1 | wake_word | wake_word | 0.898 | Correct |
| 2 | not_wake | not_wake | 0.652 | Correct |
| 3 | wake_word | wake_word | 0.711 | Correct |
| 4 | TODO | TODO | TODO | TODO |
| 5 | TODO | TODO | TODO | TODO |
| 6 | TODO | TODO | TODO | TODO |
| 7 | TODO | TODO | TODO | TODO |
| 8 | TODO | TODO | TODO | TODO |
| 9 | TODO | TODO | TODO | TODO |
| 10 | TODO | TODO | TODO | TODO |
| 11 | TODO | TODO | TODO | TODO |
| 12 | TODO | TODO | TODO | TODO |
| 13 | TODO | TODO | TODO | TODO |
| 14 | TODO | TODO | TODO | TODO |
| 15 | TODO | TODO | TODO | TODO |
| 16 | TODO | TODO | TODO | TODO |
| 17 | TODO | TODO | TODO | TODO |
| 18 | TODO | TODO | TODO | TODO |
| 19 | TODO | TODO | TODO | TODO |
| 20 | TODO | TODO | TODO | TODO |

# 7. Conclusions

The project demonstrates a complete local TinyML workflow on the XIAO ESP32S3 Sense: collecting live microphone data, training a compact neural network, quantizing it to int8 TensorFlow Lite, compiling it into Arduino firmware, and running real-time inference on the microcontroller.

The main technical issue was the mismatch between the old webcam recordings and the XIAO microphone. Training on XIAO-collected clips and replacing raw audio samples with 64 energy-envelope features made the model produce meaningful scores instead of flat 0.500 / 0.500 outputs. The deployed firmware now detects the wake phrase and sends successful HTTP pings to the PC server.

The main remaining limitation is compliance with the Report 2 class-count requirement. The current deployed version has two output classes, while the assignment asks for at least three. To make the final submission strictly compliant, the model should be extended to three classes and the live-results table should be completed with at least 20 physical-device predictions.

# Submission Checklist

Add at least one real setup photo to the document.

Complete all 20 live physical-device predictions.

For strict compliance, retrain and deploy a three-class model.

Record a short video demo if desired.